Disclaimer: If you break it, you own all the pieces.

First of all:

All of the knowledge in this modification was originated by other people: Tridge’s original source and files, d18c7db ‘s modifications and the hints from the oztivo mailing list.

This modification will provide you with a TiVo, which runs in NTSC as well as PAL mode. Of course your tuner will only support one or the other, depending on whether you start with a US TiVo with or without tuner replacement of whether you use a UK TiVo (at the time of writing this document UK TiVo’s are running 2.5.5, not 3.0).

You switch between those two modes by using the boot parameter TV_STD=PAL on a US Tivo or not. (I have not tried that, but for a UK TiVo you probably have to set TV_STD=NTSC to force it into NTSC mode – the US TiVo will default to NTSC). The component in-/outputs (the RCA and S-Video in-/outputs for a US TiVo) will support the standard selected (either PAL or NTSC).

In case you did not know, this allows to play back recordings made in one standard in the other, so the TiVo can act as a TV standards converter.

Contrary to some of the earlier patches this patch will actually allow to access all of the menus in the TiVo SW (incl. the infamous screen for setting the connections to a satellite receiver). The TiVo menu screens will think they run in NTSC mode, that means in PAL the lower part of the screen is black for the menus (since NTSC has less lines than PAL). The recordings/live TV screen however will use full PAL.

There was some discussion about inferior picture quality for PAL moded US TiVos for earlier versions. Right now I only use the Video-in inputs, and I do think the quality is very good. As for signals through a PAL tuner I can not speak and this is of course dependent on the tuner, so a general statement here will probably not be possible.

I did not work on a mod for the UK tivo, but comparing between ukchan-2.5.5.c, ustuner-2.5.5.c and the new palmod_us_tivo30.c should easily allow to do generate a version for the UK Tivo.

This is only the PAL modification. Please refer to other documents for further steps. A good start would be:

http://www.trufit.com.au/TiVo

First you will need a US Tivo running TiVo 3.0 software.

You can find images with 3.0 SW on this ftp server:

ftp://ftp.abs.net/tivo/Backups/

Please download this image and load it onto your target drive. The image will require a minimum of a 13.7Gb HDD to be used. This download is ~150meg.

We won’t be going into the how to of loading and backing up your drive, but you should also do a backup if you are going to load the new software over your old drive. There is an excellent how to on backing up and restoring your images found here http://www.newreleasesvideo.com/hinsdale-how-to/

(Note: From here onwards we will use arial bold font for things you should type. Enter them exactly as shown, except for any substutions we tell you to make. If there are quotes shown then type them too!  Stuff that is displayed as prompts or responses is in arial unbolded)

- mount /dev/hdx /mnt/tivo 
(Replace x with your TiVo drive letter. See Hinsdale howto for help on this)

- cd /mnt/tivo

- cd etc/rc.d

- cp rc.arch rc.arch.ntsc

- joe rc.arch

- Scroll down and look for the following function:

function loadMpegOutput () {

insmod /lib/modules/pxmpegdecode.o sTVStandard=$TV_STD

}

- Replace this with the following text:

function loadMpegOutput () {

echo "PALmod: in loadMpegOutput "

if [ "$TV_STD" = "PAL" ]; then

echo "PALmod: We are in PAL"

insmod -f /lib/modules/palmod_us_tivo30.o

insmod /lib/modules/pxmpegdecode.o sTVStandard=PAL

# if this is a US tivo then we need to force myworld to run as NTSC, or it crashes.

if [ $videoconfig = 3 ]; then

export TV_STD=NTSC

fi

echo "PALmod: leave routine"

else

echo "PALmod: We are in NTSC and do not change anything"

insmod /lib/modules/pxmpegdecode.o sTVStandard=$TV_STD

fi

}

- Make sure, that you update the correct name of the module you use for the PAL modification. In our example the name is: palmod_us_tivo30.o

- Copy this module into the directory: /lib/modules/

- You will either find an appropriate module in the palmod_us_tivo30.tar.gz archive or you will have to compile one. More to that later.

- Save the file and quit (ctrl-k then x)

- umount /mnt/tivo

Remove the drive from the system, and adjust the jumper if needed.

- Replace the drive back into TiVo

- Power up TiVo

- Open up a terminal emulator and set it to 9600,8,n,1 and point at appropriate serial port

- Get into the Factory Diag Menu, by pressing the Enter within 1-2 secounds after you plug the power back into TiVo, in the terminal emulator window.

- At the password: prompt, enter the password factory

- Press X for extended Menus, and then P to adjust the boot parms.

- To start the TiVo in PAL mode, at the prompt, retype what is there (this will be either /dev/hda4 or /dev/hda7, depending on what your active partition is), and add:

Old: root=/dev/hda4

New(- to abort): root=/dev/hda4 TV_STD=PAL (not necessary for a UK TiVo, it will default into PAL mod)

- To start the TiVo in NTSC mode, at the prompt, retype what is there, and for a US TiVo, make sure to delete the TV_STD. (you could use TV_STD=NTSC for a US TiVo, but it will default to NTSC. Again for a UK TiVo I would guess you have to add TV_STD=NTSC)

Old: root=/dev/hda4 ..... potentially more

New(- to abort): root=/dev/hda4

- Press Enter to continue and then B to boot TiVo.

- You should see TiVo starting in the desired Video mode. Please not, that the US TiVo will begin to start in NTSC mode in any case and switch to PAL mode, when myworld starts, so for the first few seconds you will have a flickering picture. Also the TiVo intro movie will show some stuttering and sound problems - this did not bother me, since I never let that run anyway.

Now to select the module for the PALmod.

- If you not using a PAL tuner, any of the palmod_us_tivo30.o files supplied in the archive will do the same job. Select one, rename it to the name you used in rc.arch (i.e. palmod_us_tivo30.o) and copy it to /lib/modules.

- If you are using a PAL tuner, you have to select a version, which fits the characteristics of your RF lineup.

- The file tv_chan.h is the one that has to be modified for your needs. I have several files in the archive, one for Australia (aus_chan.h), one for New Zealand (nz_chan.h) and two for continental Europe (ccir_catv_chan.h and ccir_air_chan.h). To compile a new version you need a TiVo C-Compiler (i.e. a cross compiler on a Linux box). Modify your channel header file for your needs, copy it to tv_chan.h (that is the name the C source is looking for) and type make all . Check the Makefile for your Compiler path.

- In case you are lucky and live in on of the already supplied zones, just pick one of the attached palmod_us_tivo30.o file, i.e. palmod_us_tivo30_ccir_catv.o, rename it to the name you used in rc.arch and copy it to /lib/modules

That's it -

STS